Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- Digital Watchdog®
	16220 Bloomfield Avenue,

- Cerritos, California 90703 USA

- Phone: +1 888 446-3593

- Web: www.digital-watchdog.com

- E-mail: dw-tech@dwcc.tv

DW SPECTRUM® IP VIDEO MANAGEMENT SYSTEM

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY (MasterFormat 2016)

28 20 00 	Video Surveillance

28 23 13	Video Surveillance Management System

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 13 - Video Surveillance Control and Management Systems

DW SPECTRUM® IP VIDEO MANAGEMENT SYSTEM

- GENERAL

  - SUMMARY

    - The section includes an IP Video Management System.

    - Product - DW Spectrum® IPVMS is an elegantly easy cutting-edge approach to video surveillance, addressing the primary limitations of managing enterprise-level HD video while offering the lowest total cost of deployment and ownership on the market. DW Spectrum® IPVMS includes DW® Cloud™ to easily manage all your systems and users from a simple webpage.
DW Spectrum® IPVMS is a perfect solution for applications that require ease of use, speed, efficiency and unprecedented image quality.

    - General Requirements

      - The specified product shall be an open video platform designed for use in any IP 
video application.

      - The specified software shall include, free of charge, any API or SDKs necessary to integrate 3rd party devices and systems.

      - The specified Video Management solution's architecture should include Desktop, Server, Mobile, and Cloud applications.

## Related Requirements

      - 28 05 03 - Safety and Security Network Communications Equipment

      - 28 05 15 - Servers for Electronic Safety and Security

      - 28 05 17 - Workstations for Electronic Safety and Security

      - 28 06 20 - Schedules for Video Surveillance

      - 28 21 00 - Surveillance Cameras

  - REFERENCES

    - General Abbreviations, acronyms, and standards

      - ACC: Video Codec “Active Content Compression

      - ADDS: Active Directory Domain Services

      - AGC: Automatic gain control

      - API: Application Programming Interface

      - AVI: Audio Video Interleave

      - Bit Rate: The number of bits/time unit sent over a network

      - DHCP: Dynamic Host Configuration Protocol

      - FPS: Frames per Second

      - FTP: File Transfer Protocol

      - GbE: Gigabit Ethernet (1000Mbps)

      - 264/5 (Video Compression Format)

      - HTTP: HyperText Transport Protocol

      - IEEE 802.1x: Authentication framework for network devices

      - IP: Internet Protocol

      - JPEG: Joint Photographic Experts Group (image format)

      - LAN: Local Area Network

      - MJPEG: Motion JPEG

      - MKV: Matroska video format

      - MP4: MPEG Layer-4 Audio

      - MPEG: Moving Picture Experts Group

      - NTP: Network Time Protocol

      - NTSC: National Television System Committee – a color encoding system based on 60Hz

      - ONVIF: Global standard for the interface of IP-based physical security products

      - PoE: Power over Ethernet (IEEE 802.3af/at) standard for providing power over network cable

      - PTZ: Pan/Tilt/Zoom

      - SDK: Software Development Kit

      - RAID: Redundant Array of Independent Disks

      - RTSP: Real-Time Streaming Protocol

      - RADASS: Resolution and Algorithmic Data Adaptive Scaling System

      - SMTP: Simple Mail Transfer Protocol

      - SSL: Secure Sockets Layer

      - TCP: Transmission Control Protocol

      - TLS: Transport Layer Security

      - Unicast: Communication between a single sender and a single receiver on a network

      - VMS: Video Management System

    - Definitions

      - E-mapping – Images that can be used as background in the client software to show server and camera placement at a site.

      - Media Server – A server/controller for cameras and network connections including client connections and camera stream encoding to hard disks.

      - Motion sensitivity – Degree of pixelization change in a selected area of the video scene required to trigger a recording or alarm.

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

  - SUBMITTALS

    - Product Documentation

      - Complete product and technical data specification sheets that include all material and equipment used on this project shall be included in the submitted solution proposal.

    - System Documentation

      - The System Integrator will provide:

        - List of all equipment with part numbers, manufacturer, firmware, and assigned IP addresses.

        - Locations and details for all components to be installed under this scope of work.

    - System Planning

      - Placement Diagram - the System Integrator will provide a placement diagram showing the proposed location of all system hardware devices.

      - System Calculation - the System Integrator will provide a calculation of all network bandwidth and storage requirements for System Servers to ensure proper planning of computing and 
networking infrastructure.

    - Qualifications

      - The manufacturer shall have a minimum of five (5) years’ experience in producing IP video equipment and software.

      - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, commission, and provide ongoing support for the solution.

  - ONGOING SUPPORT AND WARRANTY

    - Integrator Warranty

      - The security system DW Spectrum® IPVMS software and labor furnished by the integrator including wiring, software, hardware and third-party products shall be fully warranted for parts, materials and labor for a minimum of 1 year from date of the final acceptance of the Video Surveillance System.

    - Software Licensing and Warranty

      - Software licensing should be on a per device basis (e.g. 1 x license for 1 IP Camera or I/O device) with no base license for additional features or capabilities.

      - The DW Spectrum® IPVMS Software should be completely free for discovering compatible IP cameras and devices, live streaming or playback of offline media files (images, videos).

      - Software upgrades shall be provided by the Manufacturer without cost and without the need for an annual maintenance agreement.

  - SUPPORT

    - Lifetime software upgrades shall be provided by the Manufacturer without cost and without annual maintenance agreements.

END OF SECTION

- PRODUCT OVERVIEW

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@dwcc.tv

    - Models: DW-SPECTRUMLSC

    - Alternates:

      - DW Spectrum® IPVMS licenses

        - DW-SPECTRUMLSC001	Single (1) DW Spectrum® IPVMS recording license

        - DW-SPECTRUMLSC004	Four (4) DW Spectrum® IPVMS recording licenses

        - DW-SPECTRUMLSC010	Ten (10) DW Spectrum® IPVMS recording licenses

        - DW-SPECTRUMLSC020	Twenty (20) DW Spectrum® IPVMS recording licenses

        - DW-SPECTRUMLSC050	Fifty (50) DW Spectrum® IPVMS recording licenses

        - DW-SPECTRUMLSC100	One Hundred (100) DW Spectrum® IPVMS recording licenses

      - DW Spectrum® IPVMS analog-to-IP encoder licenses

        - DW-SPCP04LSC004	Four (4) DW Spectrum® analog-to-IP encoder licenses

        - DW-SPCP16LSC016	Sixteen (16) DW Spectrum® analog-to-IP encoder licenses

      - DW Spectrum® IPVMS I/O Module licenses

        - DW-SPIOLSC001		Single (1) DW Spectrum® I/O module license

      - DW Spectrum® IPVMS Video Wall licenses

        - DW-SPVWALL1X2		Single (1) DW Spectrum® video wall license

  - DW SPECTRUM® IPVMS OVERVIEW

    - DW Spectrum® IPVMS Software Components.

      - The System shall be comprised of four (4) applications which work together to form a 
seamless system.

        - Cloud – a cloud application that enables simple remote connectivity, viewing, and management of an unlimited number of systems and;

        - Server – a media server responsible for discovering, connecting to, and managing system users, devices, and associated;

        - Desktop – a desktop application capable of acting as a stand-alone media player or as a client application for connecting to and managing;

        - Mobile – a mobile application for iOS and Android devices that allows users to connect to, view, search, and control IP cameras over Wifi or Data.

    - DW Spectrum® IPVMS Developer and Integration Tools.

      - The DW Spectrum® IPVMS shall have built-in developer tools which are accessible from any System Server's Web Admin Interface (compatible with all major browsers) and should include, at a minimum:

        - A Generic Events Generator - a tool that helps build HTTP Generic Event calls, a method of sending events from 3rd party systems to the DW Spectrum® IPVMS, which can be used to trigger system actions in the DW Spectrum® IPVMS.

        - Server API – A REST HTTP(S) API that gives developers the ability to develop integrations to 3rd party devices and software using coding languages and frameworks of their choice.

        - API Change Log – list of breaking changes in API from version to version.

        - Video Source Integration SDK – provides the ability to integrate virtually any live or recorded video source (IP Cameras, NVRs, DVRs, etc.) into the DW Spectrum® IPVMS with methods for discovering, displaying, analyzing and recording video, as well as integrating device I/O ports and related motion detection information.

        - Storage SDK – provides the ability to integrate potential storage into System. It allows developers to read from or write to any storage location: local, remote, and even cloud one. Creating a storage plugin requires implementing standard functions such as I/O stream, if file exists, delete file, list of files in the folder, etc. Storage SDK also contains an example of using an FTP server as a storage location.

        - Metadata SDK – provides the ability to rapidly integrate 3rd party video and data analytics solutions by creating and installing Plugins on the Server application. Capabilities include requesting frames from the DW Spectrum® IPVMS in yuv420, argb, abgr, rgba, bgra, rgb and bgr formats, providing object size and coordinates, and returning metadata and events which can then be displayed in the system’s client interfaces.

    - System Architecture

      - The DW Spectrum® IPVMS shall have a Server Hive Architecture wherein:

        - All servers in a system are equal and synchronize system databases in real-time.

        - A user can connect to any system server to see and manage the entire system.

        - Servers support automatic camera fail-over to ensure a limited loss of video recording in the event of hardware or network failure.

        - Servers will use SQLite - a database technology - included in the installation package and will require no prerequisite software other than a supported operating system.

      - The DW Spectrum® IPVMS shall support one-click system-wide updates.

        - System Administrators shall be able to upgrade an entire system via a single button in the Desktop Application.

        - System Administrators shall be able to upgrade on demand to the latest release or specific builds with specific functionality or bug fixes.

        - System Administrators shall be able to apply an OTA (over-the-air) update.

        - System Administrators shall be able to generate a URL to download a portable system-specific update package in .zip file format which can be used to update servers without an active Internet connection.

      - The DW Spectrum® IPVMS will use secure technologies for inter-application communication 
and security.

        - OpenSSL for network connections - deprecated and insecure protocols and use only TLS v1+.

        - Server -> Client (Mobile, Desktop, Web) Communications - HTTPS Email - TLS / SSL - TLS is the default option for the Email Server.

        - Salted/Hashed Passwords - Local Credentials will be protected using a salted MD5 hash, Cloud Credentials should use a complex multi-level hash.

        - Encrypted video and communications between DW Spectrum® IPVMS Server and Client (Desktop, Mobile, Web) interfaces.

      - The DW Spectrum® IPVMS will not require any licenses to increase the number of supported devices, users, or servers or to support automatic camera fail-over.

  - DW SPECTRUM® IPVMS SERVER APPLICATION

    - DW Spectrum® IPVMS Server Supported Operating Systems:

      - Microsoft® Windows®:

        - Windows® 7

        - Windows® 8.1

        - Windows® 10 Home

        - Windows® 10 Pro

        - Windows® 10 Enterprise

        - Windows® Server 2008

        - Windows® Server 2008 R2

        - Windows® Server 2012

        - Windows® Server 2012 R2

        - Windows® Server 2016

        - Windows® Server 2019

      - Ubuntu® Linux®:

        - Ubuntu® 16.04 LTS: "Xenial Xerus"

        - Ubuntu® 18.04 LTS: “Bionic Beaver”

    - DW Spectrum® IPVMS Server Application Minimum Compatible / Supported Computing Hardware.

      - The DW Spectrum® IPVMS will be capable of running on any computing hardware able to run a compatible operating system and that has, at a minimum:

        - 1GB of RAM

        - 100 Mbps NIC

        - Intel® Dual-Core Atom Processor or equivalent compatible CPU

      - The DW Spectrum® IPVMS Server application will be written in such a way that it will be capable of running 128 dual-streaming IP cameras (256 total streams) on a single core of an Intel® Core i3 CPU.

    - DW Spectrum® IPVMS Server Application Initial Installation and Setup.

      - The DW Spectrum® IPVMS Server application should install without the need for any prerequisite software (proprietary databases or 3rd party software) other than a supported operating system.

      - The DW Spectrum® IPVMS Server installer package (and any dependencies, if needed) should not exceed 100 MB (megabytes).

      - The DW Spectrum® IPVMS installation process should require no user input once initiated.

      - After installation is complete the DW Spectrum® IPVMS Server setup process will allow system administrators to create a new system or to merge the newly installed server(s) to an 
existing system.

    - DW Spectrum® IPVMS Server Application Features

      - The DW Spectrum® IPVMS Server Application shall automatically discover, stream, and record any ONVIF Profile S IP camera or encoder located on the same subnet as the server application and will also discover and stream video and audio from connected USB Webcams, compatible I/O devices, and compatible Audio devices.

      - The DW Spectrum® IPVMS Server Application shall allow administrators to manually discover, stream, and record RTSP, HTTP, or UDP (multicast, unicast) streams.

      - The DW Spectrum® IPVMS Server application shall support up to 1000 concurrent 
TCP connections.

      - The DW Spectrum® IPVMS Server application shall record and stream video of any resolution and frame rate, limited only by hardware.

      - The DW Spectrum® IPVMS Server application shall support automatic camera fail-over without any additional licenses.

      - The DW Spectrum® IPVMS Server application will support an unlimited number of users and custom user roles.

      - The DW Spectrum® IPVMS Server application shall support any type of storage medium - HDD's, SSD's, SD cards, DAS, NAS, or other network-attached storage devices or locations.

      - The DW Spectrum® IPVMS Server application shall support LDAP / Active Directory / Open LDAP integration for user login credential management.

      - The DW Spectrum® IPVMS Server application shall record and stream H.264, H.265, and MJPEG streams.

      - The DW Spectrum® IPVMS Server application shall record and stream AAC, PCM (Mu-Law, 
A-law), g726, and MP3 audio.

      - The DW Spectrum® IPVMS Server application shall support transcoding streams on-demand for delivery to 3rd party systems or devices in H.265, H.264, MJPEG or WebM codecs.

      - The DW Spectrum® IPVMS Server application shall be able to provide pass-through high or low-res HLS streams from connected devices.

      - The DW Spectrum® IPVMS Server application shall store archive indices in the same location as recorded video files.

      - The DW Spectrum® IPVMS Server application shall allow system administrators to recover archives from any storage medium using a re-index archive feature.

      - The DW Spectrum® IPVMS Server application will contain a Boolean events engine allowing operators to program and trigger system actions based on system, connected device, or HTTP events sent from 3rd party system or device.

      - The DW Spectrum® IPVMS Server application shall be able to send HTTP PUT or GET requests to 3rd party systems or devices.

      - The DW Spectrum® IPVMS Server application shall support IPv4 or IPv6 addressing

      - The DW Spectrum® IPVMS Server application shall allow operators to set custom network routing configurations for system servers to optimize network routing and usage.

      - The DW Spectrum® IPVMS Server application shall allow operators to monitor the CPU, RAM, NIC, and HDD usage in real-time.

      - The DW Spectrum® IPVMS Server application shall track all operator actions to allow audits.

      - The DW Spectrum® IPVMS Server application shall generate automatic crash files every time there is an unexpected crash of the Server application.

      - The DW Spectrum® IPVMS Server application shall allow operators to change the size of reserved disk space for storage drives.

      - The DW Spectrum® IPVMS Server application shall automatically disable any system drive (drive containing the operating system) in computing hardware with more than one drive to ensure the operating system drive does not become full.

      - The DW Spectrum® IPVMS Server application shall support configuration and events from binary I/O contacts on supported devices - including IP cameras and I/O devices.

      - The DW Spectrum® IPVMS Server application shall support sending email notifications via SMTP using TLS, SSL or unsecured connections.

      - The DW Spectrum® IPVMS Server application shall support scheduled backup of recording archives to local, networked, or cloud storage locations.

      - The DW Spectrum® IPVMS Server application shall allow on-demand backup of recording archives to local, networked, or cloud storage locations.

      - The DW Spectrum® IPVMS Server application shall allow concurrent-recording of all connected cameras / streams to two (2) servers in real-time.

      - The DW Spectrum® IPVMS Server application will allow server-side, CPU-based motion analysis for all connected IP cameras with no perceptible increase (<3%) in CPU usage.

      - The DW Spectrum® IPVMS Server application will require no dedicated GPU in order to perform at maximum capacity.

      - The DW Spectrum® IPVMS Server application will have a web administration interface that allows users to view live or recorded video from a single camera at a time in high or low resolutions.

      - The DW Spectrum® IPVMS Server application will have a web administration interface that allows system administrators to view real-time server health monitoring statistics (CPU, NIC and 
HDD usage).

      - The DW Spectrum® IPVMS Server application will have a web administration interface that allows operators to disconnect the DW SPECTRUM® IPVMS Server from the DW SPECTRUM® IPVMS cloud application.

      - The DW Spectrum® IPVMS Server application will have a web administration interface that allows users to view all available servers in the system.

      - The DW Spectrum® IPVMS Server application will have a web administration interface that allows operators to switch between server interfaces.

      - The DW Spectrum® IPVMS Server application will have a hidden advanced page that gives system administrators the ability to modify advanced system settings.

      - The DW Spectrum® IPVMS Server application will support any RAID configuration of 
storage medium.

      - The DW Spectrum® IPVMS Server will support simple discovery and addition (authenticated or unauthenticated) of SAMBA NAS devices.

      - The DW Spectrum® IPVMS Server application will allow administrators to configure custom network routing rules for the system to optimize for limited network environments.

      - The DW Spectrum® IPVMS Server application will be able to send and receive custom HTTP or HTTPS events for use in syntactic integrations with 3rd party hardware and software.

  - DW SPECTRUM® IPVMS DESKTOP APPLICATION

    - DW Spectrum® IPVMS Desktop Application Supported Operating Systems:

      - Microsoft® Windows®:

        - Windows® 7

        - Windows® 8.1

        - Windows® 10 Home

        - Windows® 10 Pro

        - Windows® 10 Enterprise

        - Windows® Server 2008

        - Windows® Server 2008 R2

        - Windows® Server 2012

        - Windows® Server 2012 R2

        - Windows® Server 2016

        - Windows® Server 2019

      - Ubuntu® Linux®:

        - Ubuntu® 16.04 LTS: "Xenial Xerus"

        - Ubuntu® 18.04 LTS: “Bionic Beaver”

      - Apple® Mac OS

        - MacOS 10.11: "El Capitan" (October 2015)

        - MacOS 10.12: "Sierra" (September 2016)

        - MacOS 10.13: "High Sierra" (September 2017)

        - MacOS 10.14: "Mojave" (September 2018)

    - DW Spectrum® IPVMS Desktop Application Minimum Compatible / Supported Computing Hardware:

      - The DW Spectrum® IPVMS will be capable of running on any computing hardware able to run a compatible operating system and that has, at a minimum:

        - a CPU that supports OpenGL 2.1

        - Intel® HD Graphics 3000

      - The DW Spectrum® IPVMS Desktop application shall not require any dedicated graphics card to work at full capacity (64 streams on a 64 bit OS), shall use the CPU for all video decoding 
and rendering.

    - DW Spectrum® IPVMS Desktop Application Initial Installation and Setup:

      - The DW Spectrum® IPVMS Client application installer should not exceed 100 MB (megabytes).

      - The DW Spectrum® IPVMS Client application should be a publicly available, free download.

      - The DW Spectrum® IPVMS Client application should require no prerequisite proprietary or 3rd party software and database technologies during installation.

      - The DW Spectrum® IPVMS Client installation process should require no user input once initiated.

    - DW Spectrum® IPVMS Desktop Application Features:

      - The DW Spectrum® IPVMS Desktop application will have the following basic structure:

        - Navigation Panel - with a main menu button, an interactive cloud-login icon, tabbed layouts, minimize and maximize icons, a contextual help icon, and a close application.

        - Resource Panel (Left) - contains all system resources (Servers, Devices, Users, Layouts, Offline files, etc) with collapsible structure and a keyword search mechanism to allow operators to quickly search for a display live streams / cameras, offline video and image files, or any combination.

        - Notifications Panel (Right) - shows all system or rules-engine generated notifications which can be clicked on to display relevant resources in the Viewing Grid.

        - Timeline / Playback Panel (Bottom) - allows for navigation and search of recorded video files.

        - Viewing Grid (Main Viewing Area) - a flexible adaptive grid interface that allows operators to create and share customized layouts of system.

      - The DW Spectrum® IPVMS Desktop application shall allow operators to view and interact with the following types of media:

        - Live Streams: H.265, H.264, MJPEG.

        - Offline Media: AVI MKV MP4 MOV TS M2TS MPEG MPG FLV WMV 3GP JPG PNG GIF BMP TIFF.

        - I/O Devices: Status and Triggers.

        - Servers: Real-Time Server Health Monitoring Status.

      - The DW Spectrum® IPVMS Desktop application shall allow the operator to scroll to zoom in to any part of the Viewing Grid.

      - The DW Spectrum® IPVMS Desktop application shall allow the operator to drag & drop to reassign cameras from one server to another server.

      - The DW Spectrum® IPVMS Desktop application will have a flexible timeline that allows operators to view the dates of any and all archived video in the System for a specific camera, or groups 
of cameras.

      - The DW Spectrum® IPVMS Desktop application will allow operators to manually create bookmarks - with a start time, end time, name, description, and tags - for later search. Bookmarks shall also be able to be created using the Rules Engine.

      - The DW Spectrum® IPVMS Desktop application shall allow operators to create Soft Triggers - programmable, customizable buttons that sit on top of streams in the Viewing Grid - to trigger any available system action.

      - The DW Spectrum® IPVMS Desktop application shall have icons located on the top of live camera streams which allow operators to dewarp fisheye cameras, control PTZ cameras, apply client-side image enhancement, execute smart motion search, create zoom windows, rotate items to any orientation, and activate stream or file info.

      - The DW Spectrum® IPVMS Desktop application shall allow operators to create Zoom Windows (up to 63 zoom windows on a single item in a 64 bit OS) - a magnified view of a part of a live stream, recorded videos, or static images.

      - The DW Spectrum® IPVMS Desktop application shall allow operators the ability to execute a Smart Motion search by selecting a subset of a live camera stream with results shown in red on the flexible timeline. Smart Motion search should be able to search a year (12 months, 365 days) of archived video in less than one (1) second.

      - The DW Spectrum® IPVMS Desktop application will allow users to search live cameras by name, manufacturer, IP address, MAC address, and status (e.g. live).

      - The DW Spectrum® IPVMS Desktop application shall allow operators to search video archives by date and time with a responsive, adaptive timeline.

      - The DW Spectrum® IPVMS Desktop application will allow operators to customize the background image of the application with supported image types.

      - The DW Spectrum® IPVMS Desktop application will support digital mapping by allowing operators to add and customize background images - including opacity and number of grid items to be displayed by default.

      - The DW Spectrum® IPVMS Desktop application will utilize adaptive scaling technology to automatically switch between high and low-resolution streams during live and recording playback to optimize CPU and network usage.

      - The DW Spectrum® IPVMS Desktop application will allow operators to log in to the Cloud application in order to quickly connect to any shared system.

      - The DW Spectrum® IPVMS Desktop application will allow operators to quickly switch between previously connected or cloud-connected systems using searchable tiles that show system name and status.

      - The DW Spectrum® IPVMS Desktop application will have a Storage Analytics feature allowing operators to analyze storage capacity of the system based on available drives and real-time and historical bandwidth analysis.

      - The DW Spectrum® IPVMS Desktop application will allow management and configuration of all System devices, users, and resources in a single unified interface.

      - The DW Spectrum® IPVMS Desktop application will allow fast-forward and fast-reverse of archived video up to 16x normal speed.

      - The DW Spectrum® IPVMS Desktop application will show operators which system server they are connected to.

      - The DW Spectrum® IPVMS Desktop application will allow operators to connect to previous versions by automatically downloading and switching to compatible versions.

      - The DW Spectrum® IPVMS Desktop applications will automatically discover available systems on the same network as the computer running the Desktop application.

      - The DW Spectrum® IPVMS Desktop application will automatically recover and reconnect to a system in the instance the server the operator is connected to become inaccessible for 
any reason.

      - The DW Spectrum® IPVMS Desktop application will allow operators to show or hide adaptive thumbnails in the timeline / playback panel.

      - The DW Spectrum® IPVMS Desktop application will allow operators to synchronize all items on a layout or disable synchronization to view live and recorded video at the same time.

      - The DW Spectrum® IPVMS Desktop application will have adaptive resolution settings dialog, allowing operators to switch resolution settings while the dialog is open by clicking on Auto (Default),  High or Low.  a

      - The DW Spectrum® IPVMS Desktop application will allow batch configuration of camera recording schedules, fps and quality.

      - The DW Spectrum® IPVMS Desktop application will allow operators to drag and drop multiple system resources onto the Viewing Grid at the same time.

      - The DW Spectrum® IPVMS Desktop Application will allow administrators to modify time synchronization settings for the system to utilize online resources (NTP servers) or to set a dedicated local time server.

      - The DW Spectrum® IPVMS Desktop Application will allow system administrators to view a full list of system cameras and devices in a single dialog.

      - The DW Spectrum® IPVMS Desktop application will allow operators to view, search and export all system events.

      - The DW Spectrum® IPVMS Desktop application will allow operators to view, search and export all system bookmarks.

      - The DW Spectrum® IPVMS Desktop application will allow operators to view, search, and export system logs.

      - The DW Spectrum® IPVMS Desktop application will allow operators to view, search, and export an audit trail of all operator actions and replay related video.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to backup and restore the system database.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to create an unlimited number of custom user roles.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to create and share lockable layouts.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to update layouts in 
real-time.

      - The DW Spectrum® IPVMS Desktop application will allow users to record their screen in full resolution and up to 30 fps.

      - The DW Spectrum® IPVMS Desktop application will allow users to add a local folder to add local files for search and playback.

      - The DW Spectrum® IPVMS Desktop application will have a Video Wall mode which will allow operators to control the application remotely.

      - The DW Spectrum® IPVMS Desktop application will have a Media Player mode which will allow operators to use the application as a media player.

      - The DW Spectrum® IPVMS Desktop application will remember past system connections and user credentials and will allow operators to quickly search for and switch between systems.

      - The DW Spectrum® IPVMS Desktop application will allow operators to adjust the aspect ratio and streaming quality (high resolution or low resolution) of items displayed on the viewing grid.

      - The DW Spectrum® IPVMS Desktop application will display I/O devices as an individual item on the viewing grid and allow operators to create custom names for inputs and outputs.

      - The DW Spectrum® IPVMS Desktop application will allow users to customize the layout of I/O panels on the item in the viewing grid including indicators for inputs and buttons for outputs.

      - The DW Spectrum® IPVMS Desktop application will allow users to dewarp any fish-eye lens using automatic calibration or manual calibration without the need for any 3rd party SDKs.

      - The DW Spectrum® IPVMS Desktop application will allow users to create fully customizable viewing tours which include any combination of live video streams, offline videos, images, websites (or URLs), I/O devices, and Server health monitoring status.

      - The DW Spectrum® IPVMS Desktop application will allow system administrators to modify and save a shared layout to affect an instantaneous change to that layout on the 
DW Spectrum® IPVMS Desktop application of any user connected to the system viewing that layout (when the system administrator saves the layout will update in real-time for any user viewing that layout).

      - The DW Spectrum® IPVMS Desktop application will support two-way audio between operators and supported devices.

      - The DW Spectrum® IPVMS Desktop application will support audio alerts as an action that can be played on users’ computers or connected system devices.

      - The DW Spectrum® IPVMS Desktop application will support PTZ presets and PTZ tours.

      - The DW Spectrum® IPVMS Desktop application will support PTZ presets and tours in fish-eye cameras using dewarp mode.

      - The DW Spectrum® IPVMS Desktop application will allow operators to schedule a recording for connected cameras and devices with options to force a minimum or maximum storage duration.

      - The DW Spectrum® IPVMS Desktop application will allow operators to configure pre and post-recording for motion events.

      - The DW Spectrum® IPVMS Desktop application will allow operators to optimize camera streaming quality from connected devices automatically using low, medium, high, best quality selectors or manually in the camera.

      - The DW Spectrum® IPVMS Desktop application will allow users to export video by selecting a camera or cameras, then selecting an area on the timeline, right-clicking, and choosing export in the context menu.

      - The DW Spectrum® IPVMS Desktop application will support single video export in .avi, .mp4, or .mkv formats and will offer the option to transcode any client-side effects (image enhancement, dewarping and timestamps) as part of exported video.

      - The DW Spectrum® IPVMS Desktop application will support multi-video export in an executable format to create a fully portable version of the DW Spectrum® IPVMS Desktop application including all exported video files.

      - The DW Spectrum® IPVMS Desktop application shall have a rapid review export feature that will allow operators to compress any length of video into a short video (e.g. export 8 hours of archives into a 30-second video clip).

      - The DW Spectrum® IPVMS Desktop application shall allow system administrators to activate or deactivate system licenses on Internet-connected systems.

      - The DW Spectrum® IPVMS Desktop application shall allow users to force open an alarm layout triggered by any system or 3rd party event with one or many associated cameras or system resources (including web pages, images, offline videos, I/O devices, etc.).

      - The DW Spectrum® IPVMS Desktop application will have a hidden configurable method of increasing the number of items allowed on the viewing grid.

      - The DW Spectrum® IPVMS Desktop application shall allow users to adjust the configuration of connected IP video devices (IP cameras, I/O devices, encoders, etc.).

      - The DW Spectrum® IPVMS will now allow analytics from DW®, Axis, Hikvision, Dahua, and any device supporting ONVIF Analytics Events.

      - The DW Spectrum® IPVMS Desktop application will force users to set the camera’s initial password upon enrollment for optimal cybersecurity.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to configure installed plugins for 3rd party video analytics solutions and display detected objects in the Notifications panel.

      - The DW Spectrum® IPVMS Desktop application will allow administrators to configure the security of system connections (HTTP, HTTPS) and enable encrypted video and communications between the Server application and system clients (Desktop, Mobile, Web).

      - The DW Spectrum® IPVMS Desktop application will support advanced PTZ for supported cameras that allow operators to draw a rectangle to zoom to a specific point of interest and to double-click to zoom to original zoom parameters.

  - DW SPECTRUM® IPVMS MOBILE APPLICATION

    - DW Spectrum® IPVMS Mobile Application Supported Operating Systems:

      - Google® Android® OS

        - Android® 5.0: "Lollipop" (November 2014)

        - Android® 6.0: "Marshmallow" (October 2015)

        - Android® 7.0, 7.1: "Nougat" (August 2016)

        - Android® 8.0, 8.1: "Oreo" (August 2017)

        - Android® 9.0: "Pie" (August 2018)

      - Apple® iOS (iPhone, iPad)

        - iOS 5

        - iOS 6

        - iOS 7

        - iOS 8

        - iOS 9

        - OS 10

        - OS 11

    - DW Spectrum® IPVMS Mobile Download and Installation:

      - The DW Spectrum® IPVMS Mobile application will be available as a free download from Google Play or Apple iTunes stores.

    - DW Spectrum® IPVMS Mobile Features:

      - The DW Spectrum® IPVMS Mobile application will automatically discover available Systems on a local area network (LAN).

      - The DW Spectrum® IPVMS Mobile application will store past system connections and credentials and will allow users to quickly search for switch between systems.

      - The DW Spectrum® IPVMS Mobile application will have adaptive streaming and automatically adjust the stream being displayed based on network speed.

      - The DW Spectrum® IPVMS Mobile application will allow users to adjust streaming resolutions manually.

      - The DW Spectrum® IPVMS Mobile application will allow users to search for cameras by name.

      - The DW Spectrum® IPVMS Mobile application will allow fish-eye dewarping of any fish-eye lens without the need for any 3rd party SDK.

      - The DW Spectrum® IPVMS Mobile application will allow users to view live video from one system.

      - The DW Spectrum® IPVMS Mobile application will allow users to log in to the DW Spectrum® IPVMS Cloud layer in order to view and access all systems shared with a user.

      - The DW Spectrum® IPVMS Mobile application will utilize a custom media player to render and display live thumbnails and videos.

      - The DW Spectrum® IPVMS Mobile application will allow users to search for video using 
a calendar.

      - The DW Spectrum® IPVMS Mobile application will allow users to search for video using a 
flex timeline.

      - The DW Spectrum® IPVMS Mobile application will allow users to search for cameras by layout.

      - The DW Spectrum® IPVMS Mobile application will allow users to control devices and system actions with Soft Triggers.

      - The DW Spectrum® IPVMS Mobile application will allow users to search using Smart Motion search by selecting all or part of a camera view.

  - DW SPECTRUM® IPVMS CLOUD APPLICATION

    - DW Spectrum® IPVMS Cloud Supported Browsers:

      - The DW Spectrum® IPVMS Cloud application will allow users to log in from any modern web browser (Google® Chrome, Mozilla® Firefox, Microsoft® Edge, Opera®, etc.) from any type of device (mobile, pc, etc.).

    - DW Spectrum® IPVMS Cloud Connection Setup:

      - The DW Spectrum® IPVMS Cloud application will allow System Owners to connect an unlimited number of Systems to a single user account.

      - The DW Spectrum® IPVMS Cloud application will allow any System with an active internet connection to connect the DW Spectrum® IPVMS Cloud service which will then make the system accessible from anywhere, anytime without the need for port forwarding or dynamic DNS services.

    - DW Spectrum® IPVMS Cloud Features:

      - The DW Spectrum® IPVMS Cloud application will be an optional add-on to the DW Spectrum® IPVMS requiring no additional licensing.

      - The DW Spectrum® IPVMS Cloud application will allow users to connect an unlimited number of systems to a single user account.

      - The DW Spectrum® IPVMS Cloud application will allow system administrators to share access to a system using only an email address.

      - The DW Spectrum® IPVMS Cloud application will allow system administrators to assign custom user roles when sharing system access.

      - The DW Spectrum® IPVMS Cloud application will allow users to quickly search for and connect to cloud-connected systems by name.

      - The DW Spectrum® IPVMS Cloud application will allow operators to view live or recorded video from one camera at a time on any cloud-connected system.

      - The DW Spectrum® IPVMS Cloud application will first attempt a direct connection to system servers using NAT Traversal technology and will be able to proxy traffic to ensure access to a system in the case of ISP or routing issues.

      - The DW Spectrum® IPVMS Cloud application will allow an unlimited number of connected users and systems with no additional licensing.

      - The DW Spectrum® IPVMS Cloud application will utilize secure networking technologies (OpenSSL, HTTPS) and a complex Salted MD5 hash for any stored passwords.

      - The DW Spectrum® IPVMS Cloud application will include a web client that can be used to search for and view cameras in the browser.

  - MINIMUM HARDWARE REQUIREMENTS

    - Server

      - Up to 16 cameras:

        - Processor:				Intel® Quad Core Celeron or higher

        - System RAM:			4 GB

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual GigaBit (RJ-45)

      - Up to 64 cameras:

        - Processor:				Intel® i5 or higher

        - System RAM:			8 GB

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual GigaBit (RJ-45)

      - Up to 128 cameras:

        - Processor:				Intel® i7 or higher

        - System RAM:			16 GB

        - Hard drive (HDD) storage:		60 GB SSD, 7200 rpm HDD

        - Network Interface:			Dual GigaBit (RJ-45)

        - Acceptable Operating Systems:

          - Microsoft® Windows® 7/8/10

          - Microsoft® Windows® Server 2012

        - Network Interface: 			Dual GigaBit (RJ-45)

    - Client

      - Processor:				Intel® i3 4th Generation

      - Video

        - Graphics card:			Intel® HD graphics or comparable with 1GB memory

      - RAM:					4 GB

      - Storage:				500 GB HDD

      - Network Interface:			1Gbps (RJ-45)

      - Acceptable Operating Systems:

        - Microsoft® Windows® 7/8/10

        - Microsoft® Windows® Server 2008/2012

        - Mac® OS X

      - Mobile Client:

        - Acceptable Operating Systems

          - Google® Android

          - Apple® IOS

        - Mobile clients shall be readily available through device App Stores.

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

    - Contractor personnel shall be factory trained and certified to install, service and maintain the system.

  - PREPARATION

    - Camera and encoder compatibility shall be verified prior to deployment.

    - The network design and configuration shall be verified for compatibility and performance with the 
DW Spectrum® IPVMS.

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

  - INSTALLATION

    - The contractor shall follow all Manufacturer published installation procedures and guidelines.

    - Servers shall be installed in areas consistent with Manufacturer recommendations.

END OF SECTION