Illustra, the IP camera product line for Security Products, a division of Building Technologies & Solutions for Johnson Controls, is a leading manufacturer and designer of video surveillance products. Security and other industries require a full line-up of cyber secure, high-definition video devices that provide superior low-light, wide dynamic range, low network latency, multiple codecs and streams. The Pro, Flex and Essential family of Illustra IP cameras include PTZ and Fisheye devices, as well as a variety of different form-factor cameras such as Mini-Dome, Bullet, Box and Micro.

For additional information, please contact: Tyco Security Products 6 Technology Park Drive Westford, MA 01886 USA Phone: +1 978 577 4000 Email: VideoApps@tycoint.com Web: www.illustracameras.com

12MP NETWORK IP FISHEYE CAMERA (IPS12-F27-OI02)

MasterFormat 2016:

28 21 13.11	Panoramic IP Cameras

Notes to Specifier:

- Explanatory notes and comments are presented in colored text.

Notice

Please read this A&E document thoroughly and save it for future use before attempting to specify this unit. The information in this document was current when published. The manufacturer reserves the right to revise and improve its products. All specifications are therefore subject to change without notice.

Trademarks

Windows® is a registered trademark of Microsoft Corporation. PS/2® is a registered trademark of International Business Machines Corporation.

The trademarks, logos, and service marks displayed on this document are registered in the United States [or other countries]. Any misuse of the trademarks is strictly prohibited and Tyco Security Products will aggressively enforce its intellectual property rights to the fullest extent of the law, including pursuit of criminal prosecution wherever necessary. All trademarks not owned by Tyco Security Products are the property of their respective owners, and are used with permission or allowed under applicable laws.

Important Note to Security Systems Specifiers

CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written using MasterFormat 2016. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

Primary Specification Area:

MasterFormat 2016:

28 20 00	Video Surveillance

28 21 00	Surveillance Cameras

28 21 13	IP Cameras

28 21 13.11	Panoramic IP Cameras

Related Requirements:

MasterFormat 2016:

Division 27	Communications

27 20 00	Data Communications

27 21 00	Data Communications Network Equipment

27 21 29	Data Communications Switches and Hubs

Division 28	Electronic Safety and Security

28 05 00	Common Work Results

28 05 07.13	Power Sources for Video Surveillance

28 05 07.21	Poe Power Sources for Electronic Safety and Security

28 05 11	Cyber Security Requirements for Electronic Safety and Security

28 05 19.13	Hybrid Digital Video Recorders

28 05 19.15	Network Video Recorders

28 05 33	Safety and Security Network Communications Equipment

28 06 20	Schedules for Video Surveillance

28 21 19	Camera Mounts

28 23 00	Video Management System

12MP NETWORK IP FISHEYE CAMERA

- GENERAL

  - SUMMARY

- Section includes a True Day/Night with IR 12MP imager network IP indoor/outdoor white non-vandal fisheye camera with single RJ-45 for HD quality images, WDR for superior low-light performance, high-sensitivity low-noise CMOS progressive sensor, and both distortion-free edge-based and client-based dewarping without plug-in or third-party driver dependencies.

- Product: A NVT complaint ONVIF Profile S, G and Q, standard single lens camera powered by 7.4W by PoE IEEE 802.3af or 8.1W 12VDC.

- Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

- Abbreviations

- 2D/3D DNR	2-Dimensional / 3-Dimensional Digital Noise Reduction

- 12MP	Twelve MegaPixel (12,000,000 pixels)

- AEC	Automatic Exposure Control

- AGC	Automatic Gain Control

- ARP	Address Resolution Protocol

- B/W	Black and White

- BNC	Bayonet Neill Concelman connector

- CAT5	Category 5 twisted pair Ethernet cable

- CAT5E	Category 5 Enhanced twisted pair Ethernet cable

- CAT6	Category 6 twisted pair Ethernet cable

- CBR	Constant Bit Rate

- CVBR	Constrained Variable Bit Rate

- CCTV	Closed-Circuit Television (aka Video Surveillance)

- CMOS	Complementary Metal-Oxide Semiconductor image sensor

- DHCP	Dynamic Host Configuration Protocol

- DNS	Domain Name System

- DDNS	Dynamic Domain Name System

- DOF	Depth of Field

- dPTZ	Digital Pan-Tilt-Zoom functionality (aka Electronic PTZ)

- DPWS	Devices Protocol for Web Services

- ePTZ	Electronic Pan-Tilt-Zoom functionality (aka Digital PTZ)

- FE	Fisheye

- FOV	Field of View

- FPS	Frames per Second

- FTP	File Transfer Protocol

- GB	Giga Bytes

- GbE	Gigabit Ethernet

- GOP	Group of Pictures

- H.264	MPEG-4 Part 10, Advanced video compression format standard

- HD	High Definition

- HTTP	Hypertext Transfer Protocol

- HTTPS	Hypertext Transfer Protocol Secure

- iAPI3	Illustra Application Programming Interface (version 3.x)

- ICMP	Internet Control Message Protocol

- ICR	Infrared Cut-filter Removal

- IETF NTP	Internet Engineering Task Force for Network Time Protocol working group

- IGMP	Internet Group Management Protocol

- IK07	Impact Protection (07 = Impact Proof from 40 cm height and 2 joules)

- IK10	Ingress Protection rating (Vandal Proof)

- IP	Internet Protocol

- IP66	Ingress Protection (66 = Dust-Tight and Protected against heavy 	seas)

- IP67	Ingress Protection (67 = Dust-Tight and Immersion up to 3 feet / 1 meter)

- IPS	Images per Second (Frames per Second)

- IPv4	Internet Protocol Version 4 (32-bit addresses)

- IPv6	Internet Protocol Version 6 (128-bit addresses)

- IR	Infrared

- LAN	Local Area Network

- LLDP	Link Layer Discovery Protocol

- Mbps	Megabits per second (10 or 100 or 1,000)

- MB	Mega Bytes

- MD	Mini-Dome form-factor

- mDNS	Multicast DNS

- Micro SDXC	Onboard Storage Media Type

- MJPEG	Motion JPEG video compression format standard

- Mbps	Megabits per second (10 or 100 or 1,000)

- mm	Millimeter

- nm	Nanometer

- n/a	Not Applicable

- NR	Noise Reduction

- NTP	Network Time Protocol

- NVT	Network Video Transmitter

- OSD	On-Screen Display

- ONVIF	Open Network Video Interface Forum

- PEAP	Protected Extensible Authentication Protocol

- PoE	Power over Ethernet standard

- PoE+	Power over Ethernet Plus standard

- PPPoE	Point-to-Point Protocol over Ethernet

- PTZ	Pan-Tilt-Zoom optical functionality

- QoS	Quality of Service

- ROI	Region of Interest

- RTCP	RTP Control Protocol

- RTP	Real-Time Transport Protocol

- RTSP	Real Time Streaming Protocol

- SDN	Soft Day/Night

- SFTP	Secure File Transfer Protocol

- SMTP	Simple Mail Transfer Protocol

- SNMP	Simple Network Management Protocol

- SOAP	Simple Object Access Protocol

- SSH	Secure Shell protocol

- SSL	Secure Socket Layer encryption

- TCP/IP	Transmission Control Protocol / Internet Protocol

- TDN	True Day/Night

- TLS	Transport Layer Security cryptographic protocol

- UDP	User Datagram Protocol

- UPnP	Universal Plus and Play

- VBR	Variable Bit Rate

- WDR	Wide Dynamic Range

- WAN	Wide Area Network

- WS	Web Services

- WS4D	Web Services for Devices

- WSDL	Web Services Description Language

- XML	Extensible Markup Language

- Zeroconf	Zero-configuration networking

- Reference Standards

- Safety Standards:

- UL 60950-1 – Information Technology Equipment (USA)

- UL 60950-22 – Information Technology Equipment (USA)

- UL 2043

- EN 60950-1 – Information Technology Equipment (EU)

- CSA 22.2 No. 60950-22-07 – Information Technology Equipment (Canada)

- IEC 60950-1 – Information Technology Equipment

- IEC 60529 – Information Technology Equipment

- Emissions Standards:

- FCC CFR 47 Part 15 Class A – Commercial or Industrial Use (USA)

- EN55022:2010 Class A – Information Technology Equipment (EU)

- EN61000-3-2:2006/A2-2009 – Information Technology Equipment (EU)

- EN61000-3-3:2008 – Information Technology Equipment (EU)

- AS/NZS CISPR 22:2009 Class A – Information Technology Equipment (Australia/NZ)

- ICES-003/NMB-003 Class A – Information Technology Equipment (Canada)

- IEEE Standards:

- 802.1x Port-based Network Access Control provides authentication mechanism

- 802.3: Ethernet standard that specifies physical media and working characteristics of Ethernet.

- 802.3af: PoE (Power over Ethernet) standard that provides 12.95 watts (maximum 15.4 watts and 250 mA current) of electrical current over a complaint Ethernet cable (CAT5e or CAT6) as an alternative power source to 24VAC power input for operating the device.

- 802.3at: PoE+ (Power over Ethernet Plus) standard that provides 25.50 watts (maximum 30.00 watts and 600 mA current) of electrical current over a complaint Ethernet cable (CAT5e or CAT6) as an alternative power source to 24VAC power input for operating the device.

- Immunity Standards:

- EN55024

- EN50130-4:2011

- Environmental Standards:

- UL-60950-22

- IEC 60529

- REACH

- RoHS

- WEEE

- Definitions

- Fisheye – A camera with an ultra-wide-angle lens inside of a small transparent bubble and sleek discreet body design that creates a hemispherical or panoramic image that provides two views:

- When ceiling mounted, it will provide a 360-degree bird’s eye view

- When wall mounted, it will provide a 180-degree panoramic view

- PoE – The IEEE 802.3af standard that provides 12.95 watts (maximum 15.40 watts and 250 mA current) of electrical current over a complaint Ethernet cable (CAT5e or CAT6) as an alternative power source to DC power input for operating the device.

- PoE+ (aka PoE Plus) – The IEEE 802.3at standard that provides 25.50 watts (maximum 30.00 watts and 600 mA current) of electrical current over a complaint Ethernet cable (CAT5e or CAT6) as an alternative power source to DC power input for operating the device.

  - SUBMITTALS

- Product Data

- Manufacturer’s printed or electronic data sheets

- Manufacturer’s installation and operation manuals

- Manufacturer’s release notes, application notes, and/or technical bulletins

  - QUALIFICATIONS

- Manufacturer shall be ISO 9001:2008 certified and a reputable Fortune 100 company with a minimum of 20 years’ experience in manufacturing for in the security industry.

- Contractor shall provide a competent trainer who has extensive experience with the installed systems and in delivering training to provide the instruction. As an alternative, the Contractor may propose the use of factory training personnel for onsite Professional Services.

  - WARRANTY

- Manufacturer shall provide a limited 3 year warranty for the device to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

- Manufacturer:	Tyco Security Products

6 Technology Park Drive

Westford, MA 01886  USA

Phone:	+1 978 577 4000

Email:	VideoApps@tycoint.com

Web:	www.illustracameras.com

- Model:		IPS12-F27-OI02

- Alternatives:	n/a

  - DESCRIPTION

- The network IP camera shall:

- Built-in web server

- Support multiple streams, up to 2992x2992 resolution

- Supports multiple dewarping modes for distortion-free panoramic views

- IR illumination effective to 15m

- Progressive scan imager

- ePTZ control with presets, scans and sequences on dewarp active images

- Onboard storage for local recording

- IK10 and IP66 rated

- Complies with ONVIF Profile S, G and Q

- Capable of integration with network recording systems which support iAPI3 or ONVIF Profiles S, G, and Q

- Capable of operation as a standalone device on the network

- Requires no plug-ins or third-party drivers

- Functions

- Video Compression

  - H.265

  - H.265 IntelliZip

  - H.264 (default)

  - H.264 IntelliZip

  - MJPEG

- ONVIF Profile S,

- Auto-Discovery

- Automatic Addressing

- Default and Reset

- GetSnapshot Uri

- Maximum AGC

- Manual Red and Blue adjustments

- Quality Priority

- SetDiscoveryMode

- SOAP over HTTP

- WS-Discovery

- Web browser access Without plug-in or third-party driver dependencies:

- Microsoft Internet Explorer (V10 and higher)

- Google Chrome

- Mozilla Firefox

- Streaming methods for audio, video and metadata:

- Unicast

- Multicast

- Dewarp Viewing Modes:

- Ceiling Mount:

- Ceiling Panorama

- Quad Active Image (4x ePTZ)

- Active Image

- Wall Mount:

- Wall Panorama

- Combination (Panorama and 2 ePTZ)

- Active Image

- When using edge-based dewarping, the 12MP resolution fisheye camera shall support up to 3 streams with the following resolution options with maximum fps dependent on resolution and viewing mode:

  - Stream 1 – H.265, H.265 IntelliZip, H.264, H.264 IntelliZip

- Maximum of 30 ips @ 2688x1512 (Panoramic 16:9) for H.264

- Maximum of 30 ips @ 720x720 (Fisheye 1:1)

- Stream 2 – H.264 or MJPEG

- Maximum of 30 ips @ 1408x1408 (Fisheye 1:1)

- Stream 3 – H.264 or MJPEG

- Maximum of 20 ips @ 960x544 (Active Image (ePTZ 16:9)

- Maximum of 30 ips @ 720x720 (Fisheye 1:1)

- Maximum Shared Streams

- Total Bandwidth up to 82,000 Kbps for Multiple Fisheye Streams

- Total Bandwidth up to 50,000 Kbps for Multiple Non-Fisheye Streams for ePTZ

Note to specifier: See attached tables for full list of resolutions.

- When using client-based dewarping (default), the 12MP resolution fisheye camera supports up to 3 streams with the following resolution options with maximum fps dependent on resolution and viewing mode:

- Stream 1 – H.264 or MJPEG

- Maximum of 30 ips @ 2992x2992 (Fisheye 1:1) for H.264

- Maximum of 30 ips @ 1408x1408 (Fisheye 1:1) for MJPEG

- Stream 2 – H.264 or MJPEG

- Maximum of 30 ips @ 1072x1072 (Fisheye 1:1)

- Stream 3 – MJPEG

- Maximum of 30 ips @ 720x720 (Fisheye 1:1)

Note: See stream table for Dewarped combinations

- Maximum Shared Streams

- Total Bandwidth up to 82,000 Kbps for Multiple Fisheye Streams

- Total Bandwidth up to 50,000 Kbps for Multiple Non-Fisheye Streams for ePTZ

- Internet Explorer for GUI stream dewarp

Note to specifier: See attached tables for full list of resolutions.

- Cyber Security and Authentication

- Enhanced Security:

  - One-Click Security Hardening

  - Certificate Management

  - Disabling Unused Protocols

  - Firewall

  - User Access Log for Cyber Audit Trail

  - Validate Complex Credentials

- IEEE 802.1x:

  - EAP-TLS – RFC 5216

  - EAPoL – RFC3748 & RFC5247

  - PEAP

- HTTPS Encryption (HTTP over TLS) – RFC2818

- IP Address Filtering

- Multi-Level Password Protection

- SSL

- TLS – RFC5346

- WS-Policy (Service Connection)

- WS-Security (ONVIF)

- Camera Characteristics:

- Onboard Storage: (Not Included)

- Type: Micro SDXC Card

- Speed: Class 10 or higher

- Capacity: Maximum of 64 GB

- Memory:

  - ROM/Flash: 128 MB

  - RAM: 8 Gbits

- Video Codec Settings:

        - H.264

- High Profile

- GOP Range: 1 to 150

- Bit Rate Methods: CBR, VBR, CVBR

- VBR Quality: Lowest, Low, Medium, High, Highest

- CBR Bit Rates: 64 to 20,480 Kbps

- CVBR Settings: Quality or Bitrate

- Frame Skip: Slow Shutter Mode

        - H.265

- High Profile

- GOP Range: 1 to 150

- Bit Rate Methods: CBR, VBR, CVBR

- VBR Quality: Lowest, Low, Medium, High, Highest

- CBR Bit Rates: 64 to 20,480 Kbps

- CVBR Settings: Quality or Bitrate

- Frame Skip: Slow Shutter Mode

        - MJPEG

- Quality: 1 to 100

- Audio Settings

              - Standard: G.711 u-law

              - Sampling Rate: 8 KHz

              - Sampling Rate: 8 bit

              - Inputs:

- Connector Type: Terminal block 2.5 mm

              - Outputs:

- Connector Type: Terminal block 2.5 mm

              - Encoding Methods:

- G.711 a-law

- G.711 u-law

- G.726

- AAC

- PCM

- Imager: 1/1.7” 12MP CMOS progressive scan

- Illuminator

- IR LEDs: 4

- IR Distance: 15 m

- Wavelength: 850 nm

- Lens:

- Format: 1/1.7”

- Design: Glass Aspherical

- Mount: C/CS 12 mm

- Focal

- Length Range: 2.10 mm

- Means: Fixed

- Focus Type: Manual

- Aperture: F 2.8

- Iris Type: Manual Iris

- IR Correction: Optical Corrective

- Day/Night: True D/N with mechanical removable IRCF

- Field of View:

- Horizontal: 	180º

- Vertical: 	180º

- Video Imaging

- Exposure Compensation Offset

- Shutter Speed:

- 1/10,000 sec to 1/1.5 sec

Note to specifier: See attached tables for full list of shutter speed.

- Back Light Compensation

- White Balance: AWB

- Wide Dynamic Range Method: Digital WDR (Off, Low, Medium, High)

- Noise Reduction:

- 2D DNR (Off, On)

- 3D DNR (Off, Low, Medium, High)

- Color Noise Reduction (Off, Low, Medium, High)

- Minimum Illumination:

- Color: 0.1 Lux

- B/W: 0.01 Lux

- Pan-Tilt-Zoom:

- ePTZ on Dewarp Active Images

- Preset configuration

- Scan configuration

- Sequence configuration

- Motion Detection:

- 4 Selectable ROI Areas

- Metadata Event Generation

- Privacy Zones: 5

- Event Alarms:

- Triggers:

- Alarm Input: 1 Set, 5V 10kΩ pull up

- Audio

- Motion Detection

- Network Loss

- Scheduled

- Tamper Alarm

- Actions:

- Auxiliary Output: 1 Set, Photo Relay 130mA /300V DC/AC

- FTP file transfer

- HTTP file transfer

- Pre-alarm recording: 10 sec.

- SD card storage

- SMTP email file transfer

- I/O Interfaces:

- Alarm Input: 1

- Alarm Output: 1

- Audio Input: 1 Bi-directional Audio

- Audio Output: 1 Bi-directional Audio

- Return to Factory Default Settings: 1 Recessed Push Button

- Menu Languages:

- English (US)

Note to specifier: See attached tables for full list of menu languages.

- User Accounts:

- Administrator

- Operator

- User

- OSD:

- Camera Name

- Date

- Time

- User Defined Text

- Electrical Characteristics:

- Input Voltage options:

- IEEE 802.af Class 1	PoE RJ45 (Priority)

- DC 12V (+/- 10%)	Terminal block 3-pin 5.0 mm

- Power Draw:

- Wattage:

- PoE: 7.1W

- DC: 8.1W

- In Rush Current: < 0.8A within 100ms

- Network Characteristics:

- Ethernet interface: 10/100/1000 Mbps Ethernet

- Ethernet port: Single Copper RJ-45

- Management

- Remote configuration and management via web browser of the following:

- System Status

- IP Address Settings

- SSL Encryption

- Password Protected User Levels

- System Log

- Device Name

- Firmware Upgrade

- Remote configuration and management via Illustra Connect of the following:

- Device Information (Device Name, Hostname, IP Address, Model Name, Product Code, Manufacturer, Firmware Version, Status, MAC Address)

- Discovery (IPv4, IPv6 or both; ONVIF, CGI IP Address or all 3)

- Snapshot

- Configure Network/Users/Date-Time (Single or Bulk)

- IP Address Settings

- Diagnostics

- Firmware Upgrade (Single or Bulk)

- Access Web Server

- Export Device List in CVS Format

- Device Management (Reset or Reboot)

- Base Protocol: TCP/IP – RFC4614

- Broadcast Protocol: UDP

- Communication Protocol: IGMP

- Data Transmissions:

- HTTP – RFC2616

- FTP – RFC959

- Discovery Methods:

- UPnP

- WS-Discovery (ONVIF and Illustra Connect)

- Email:

- SMTP – RFC5321

- Authenticated SMTP – RFC4954

- External Interface Protocol:

- iAPI3 v3.3

- ONVIF Profile S, G and Q

- SOAP 1.2

- WS-Addressing

- WS-Eventing

- File Transfer: FTP

- Message Format: XML

- Network Address Configurations:

- ARP

- DHCP – RFC2131

- IPv4 – RFC791

- IPv6 – RFC2460

- Static IP

- Zeroconf – RFC3927

- Illustra Connect Utility

- Network Layer: ICMP

- Network Management: SNMP

- Network Name Resolution:

- DNS – RFC5395

- DDNS – RFC2136

- Network Performance: QoS

- Network Protocol: PPPoE

- Streaming Methods:

- RTP – RFC3550

- RTSP – RFC2326

- RTSP over HTTP

- Unicast

- Multicast – RFC1112 Level 1

- Time Synchronization:

- NTP – RFC1305

- Poll Rate: 1 per day

- Transport Layers:

- TCP – RFC973

- UDP – RFC768

- Audio Transport: RTP/RTSP

- Video Transport: RTP/RTSP

- Web Services

- Data Object Exchange Specification: WS4D

- Description Language: WSDL

- Discovery: WS-Discovery

- Event Handling (Alarms)

- WS-Base Notification

- WS-Eventing

- WS-Topics

- Protocol: SOAP

- Security: WS-Security

- Security Permissions: WS-Policy

- Service Connection: WS-Addressing

- Specification: DPWS

- Tool Kit: WS4D

- Diagnostic Indicators

- Network

- Green LED: Ethernet Link

- Orange LED: Activity Link

- Physical:

- Dimensions (WxD):	148 mm x 57.7 mm (5.83 in x 2.27 in)

- Weight:	330 g (0.73 lbs)

- Vandal Rating:	IK10

- Environmental Rating:	IP66

- Installation Type:	Indoor or Outdoor

- Color:	White

- Bubble:	Clear

- Lens Cover:

- Transmittance:	90%

- Material Type:	Polycarbonate

- Materials:

- Housing:	Aluminum

- Bubble Trim Ring:	Polycarbonate

- Environmental:

- Cold Start Temperature: 	-25º C to 50º C (-13º F to 122º F)

- Operating Temperature: 	-30º C to 50º C (-22º F to 122º F)

- Storage Temperature: 	-15º C to 65º C (5º F to 149º F)

- Relative Humidity: 	10% to 90% (non-condensing)

  - ACCESSORIES

- Documentation:

- CD with User Manual and Illustra Connect discovery tool

- Quick Start Guide with Template

- Regulatory

- Hardware:

- Tool:	Pin Torx Security L-Key

- Mounting Screws:	3 Plastic Anchors

6 M3 Standard screws with washer

3 M3 Standard screws without washer

O-Ring

3M Tape

- Adapter Plate

- Standoff Ring

- Cable Seal

- Power Terminal Block

- Desiccant

- Mounting Adapter Types:

- Single-Gang and Double-Gang Electrical Boxes (Adapter Plate Included)

- 4s Electrical Box (Adapter Plate Included)

- Wall Mount (Optional)

- Model Number: 	ADCIM6WALLWK

- Description: 	Illustra 600 Indoor / Outdoor Wall Mount

- Pendent (Optional)

- Model Number: 	ADCI6DPCAPOW

- Description: 	Illustra White Outdoor Pendent Cap, 3/4" NPT

- Tilt Mount (Optional)

- Model Number: 	IPFETILT MOUNT

- Description: 	Illustra Tilt Mount

END OF SECTION

- EXECUTION

  - INSTALLERS

- Contractor shall comply with all applicable state and local licensing requirements.

- Contractor shall demonstrate network and CCTV knowledge to the satisfaction of the Owner’s representative.

  - EXAMINATION

- Submission of a proposal confirms that the contract documents and site conditions are accepted without qualifications unless exceptions are specifically noted.

- The site shall be visited on a regular basis to appraise ongoing progress of other trades and contractors, make allowances for all ongoing work, and coordinate the requirements of this contract in a timely manner.

- Inspect systems before installation for cosmetic defects or damage.

  - PREPARATION

- Prior to installation, the camera device and network shall be configured and tested to meet the specified requirements in accordance with the manufacturer’s instructions.

  - INSTALLATION

- Contractor shall follow all Manufacturer’s published installation guidelines for installing, programming and testing of the camera.

- In order to ensure a complete, functional camera, for bidding purposes, where information is not available from the Owner upon request, the worst-case condition shall be assumed.

- Camera placement, pointing position, and configurations shall be coordinated with the Owner’s representative, where appropriate.

- All necessary back boxes, racks, connectors, supports, conduit, cable, and wire must be furnished and installed to provide a complete and reliable camera installation. Exact location of all boxes, conduit, and wiring runs shall be presented to the Owner for approval in advance of any installation.

- All conduit, cable, and wire shall be installed parallel and square with building lines, including raised floor areas. Conduit fill shall not exceed forty percent (40%). All wires shall be gathered and tied up to create an orderly installation.

- The vandal-resistant network IP camera shall be installed within an indoor or outdoor environment and ceiling or wall mounted methods using provided or optional mounting accessory.

- Contractor ensures that the installed product contains the latest revision of Manufacturer’s firmware.

- When the camera is part of a larger security system, the camera vendor must also be able to offer Professional Services that would deploy trained personnel to assist with the installation and programming of the camera.

  - TESTING AND CERTIFICATION

- Contractor shall demonstrate the functionality of the camera upon completion of installation, documenting the result of all tests and providing these results to the Owner.

- Contractor shall provide the necessary blank forms with instructions to fill in all the required data information that shall make up the information for the installed camera devices (location, installation position, make, model, IP address, username, password, image settings).

- Upon satisfactory on line operation of the camera devices, the entire installation including all subsystems shall be inspected. Contractor shall perform all tests, furnish all test equipment and consumable supplies necessary and perform any work as required to establish performance levels for the system in accordance with the specifications. Each device shall be tested as a working component of the completed system. All system controls shall be inspected for proper operation and response.

- Tests shall demonstrate the response time and display format of each different type of input sensor and output control device. Response time shall be measured with the system functioning at full capacity. Computer operation shall be tested with the complete data file.

- Contractor shall maintain a complete log of all inspections and tests. Upon final completion of system tests, a copy of the log records shall be submitted as part of the as-built documentation along with a letter of certification to indicate that the tests have been performed, and all devices are operational.

- The system test shall be witnessed by the Authority Having Jurisdiction. Any deficiencies noted during the testing must be corrected.

- A letter of certification shall be provided to indicate that the tests have been performed, and all devices are operational.

- The Owner’s representative shall accept the system.

  - MAINTENANCE

- Preventative Maintenance Agreement during Warranty: As a separate price item, the Contractor shall provide preventative maintenance during the warranty period. Maintenance shall include, but no be limited to:

- Labor and materials, at no additional cost, to troubleshoot the camera devices.

- Labor and materials, at no additional cost, to provide test and adjustments to the camera devices.

- Regular inspections

- Preventative Maintenance Agreement: As a separate price item, the Contractor shall provide a complete Maintenance Agreement for a period of 12 months after the conclusion of the warranty period. The Maintenance Agreement shall include, but not be limited to:

- Labor and materials, at no additional cost, to troubleshoot the camera devices.

- Camera devices.

- Labor and materials, at no additional cost, to provide test and adjustments to the camera devices.

- Regular inspections.

  - STORAGE

- The product shall be stored in an environment consistent within the Manufacturer’s published environmental ratings.

  - ATTACHED TABLES

- Table 1: Shutter Speeds

| Shutter Speeds |  |  |  |
| --- | --- | --- | --- |
| 1/10,000 sec | 1/425 sec | 1/100 sec | 1/6 sec |
| 1/3,500 sec | 1/300 sec | 1/75 sec | 1/3 sec |
| 1/2,500 sec | 1/215 sec | 1/50 sec | 1/1.5 sec |
| 1/1,250 sec | 1/150 sec | 1/25 sec |  |
| 1/600 sec | 1/120 sec | 1/12 sec |  |
| Table 3 – Shutter Speed |  |  |  |

- Table 2: Menu Languages

| Menu Languages |  |  |  |
| --- | --- | --- | --- |
| Arabic | Dutch | Italian | Spanish |
| Czech | English (US) | Japanese | Swedish |
| Chinese (Simplified) | French | Korean | Turkish |
| Chinese (Traditional) | German | Polish |  |
| Danish | Hungarian | Portuguese (Brazilian) |  |
| Table 4 – Menu Languages (Default = English US) |  |  |  |

Table 3: Stream Tables

- CLIENT DEWARP

Table 3.1    Client Dewarp Stream Table

|  |  | Resolution | Max FPS |
| --- | --- | --- | --- |
|  |  | Client Dewarp |  |
| Stream 1 | h.265, h.264 | 2992x2992 | 30 |
|  |  | 2048x2048 | 30 |
|  |  | 1408x1408 | 30 |
|  |  | 1072x1072 | 30 |
|  |  | 960x960 | 30 |
|  | MJPEG | 1408x1408 | 30 |
|  |  | 1072x1072 | 30 |
|  |  | 960x960 | 30 |
| Stream 2 | h.265, h.264, | 1072x1072 | 30 |
|  |  | 960x960 | 30 |
|  |  | 720x720 | 30 |
|  |  | 640x640 | 30 |
|  |  | 480x480 | 30 |
|  |  | 384x384 | 30 |
|  | MJPEG | 1072x1072 | 30 |
|  |  | 960x960 | 30 |
|  |  | 720x720 | 30 |
|  |  | 640x640 | 30 |
|  |  | 480x480 | 30 |
|  |  | 384x384 | 30 |
| Stream 3 | MJPEG | 720x720 | 30 |
|  |  | 640x640 | 30 |
|  |  | 480x480 | 30 |
|  |  | 384x384 | 30 |

- EDGE DEWARP

| Table 3.2 Edge Dewarp Stream Table with a Fisheye view on Stream 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Image Source Max FPS |  |  |  |
| STREAM 1 |  |  | Fisheye | Fisheye | Fisheye | Fisheye |
|  | STREAM 2 |  | Fisheye | Active Image | Panorama | QUAD-CEILING Triple-WALL |
|  |  | STREAM 3 | Active Image | Active Image | Active Image | Active Image |
| 2976x2976 | 960x960 | 480x360 | 15 | 15 | N/A | 15 |
|  | 960x720 |  |  |  | 15 |  |
|  | 960x544 |  |  |  |  |  |
| 2048x2048 | 2048x2048 |  | 27 | 27 | N/A | 25 |
|  | 2048x1536 |  |  |  | 27 | 27 |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL |  |
|  | 1600x1200 |  |  |  |  | 30-CEILING 27-WALL |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL | 30 |
|  | 960x544 |  |  |  |  |  |
| 2048x1536 | 2048x2048 |  |  | 27 | N/A | 25 |
|  | 2048x1536 |  |  |  | 27 | 27 |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL |  |
|  | 1600x1200 |  |  |  |  | 30-CEILING 27-WALL |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL | 30 |
|  | 960x544 |  |  |  |  |  |
| 1920x1080 | 2048x2048 |  |  | 27 | N/A | 25 |
|  | 2048x1536 |  |  |  | 27 | 27 |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL | 30 |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL |  |
|  | 960x544 |  |  |  |  |  |

| Table 3.3 Edge Dewarp Stream Table with an Active Image on Stream 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Image Source Max FPS |  |  |  |
| STREAM 1 |  |  | Active Image | Active Image | Active Image | Active Image |
|  | STREAM 2 |  | Fisheye | Active Image | Panorama | QUAD-CEILING Triple-WALL |
|  |  | STREAM 3 | Active Image | Active Image | Active Image | Active Image |
| 2976x2976 | 960x960 | 480x360 | 15 - CEILING N/A- WALL | 15 - CEILING N/A- WALL | N/A | 15 - CEILING N/A- WALL |
|  | 960x720 |  |  |  | 15 - CEILING N/A- WALL |  |
|  | 960x544 |  |  |  |  |  |
| 2048x2048 | 2048x2048 |  | 27 | 27 | N/A | 25 |
|  | 2048x1536 |  |  |  | 27 | 27 |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL |  |
|  | 1600x1200 |  |  |  |  | 30-CEILING 27-WALL |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL | 30 |
|  | 960x544 |  |  |  |  |  |
| 2048x1536 | 2048x2048 |  |  | 27 | N/A | 27 |
|  | 2048x1536 |  |  | 30 | 27-CEILING 30-WALL |  |
|  | 1920x1080 |  |  |  |  | 30 |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL |  |
|  | 960x544 |  |  |  |  |  |
| 1920x1080 | 2048x2048 |  | 30 |  | N/A | 27 |
|  | 2048x1536 |  |  |  | 30 | 30 |
|  | 1920x1080 |  |  |  |  |  |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 30 |  |
|  | 960x544 |  |  |  |  |  |

| Table 3.4 Edge Dewarp Stream Table with Panorama view on Stream 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Image Source Max FPS |  |  |  |
| STREAM 1 |  |  | Panorama | Panorama | Panorama | Panorama |
|  | STREAM 2 |  | Fisheye | Active Image | Panorama | QUAD-CEILING Triple-WALL |
|  |  | STREAM 3 | Active Image | Active Image | Active Image | Active Image |
| 2976x2976 | 960x960 | 480x360 | N/A |  |  |  |
|  | 960x720 |  |  |  |  |  |
|  | 960x544 |  |  |  |  |  |
| 2048x2048 | 2048x2048 |  |  |  |  |  |
|  | 2048x1536 |  |  |  |  |  |
|  | 1920x1080 |  |  |  |  |  |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  |  |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  |  |  |
|  | 960x544 |  |  |  |  |  |
| 2048x1536 | 2048x2048 |  | 27 | 27 | N/A | 27-CEILING 27-WALL |
|  | 2048x1536 |  |  | 27-CEILING 30-WALL | 25-CEILING 30-WALL | 27 |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL | 27-CEILING 30-WALL |
|  | 1600x1200 |  |  |  | 25-CEILING 30-WALL | 30 |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 25-CEILING 30-WALL |  |
|  | 960x544 |  |  |  | 27-CEILING 30-WALL |  |
| 1920x1080 | 2048x2048 |  | 27-CEILING 30WALL | 27-CEILING 30-WALL. | N/A | 27 |
|  | 2048x1536 |  |  |  | 27-CEILING 30-WALL | 27-CEILING 30-WALL |
|  | 1920x1080 |  |  | 30 |  | 30 |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL |  |
|  | 960x544 |  |  |  |  |  |

| Table4 Edge Dewarp Stream Table with Quad_View_Ceiling/Triple_View_Wall on Stream 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Image Source Max FPS |  |  |  |
| STREAM 1 |  |  | QUAD-CEILING Triple-WALL | QUAD-CEILING Triple-WALL | QUAD-CEILING Triple-WALL | QUAD-CEILING Triple-WALL |
|  | STREAM 2 |  | Fisheye | Active Image | Panorama | QUAD-CEILING Triple-WALL |
|  |  | STREAM 3 | Active Image | Active Image | Active Image | Active Image |
| 2976x2976 | 960x960 | 480x360 | 15 | 15 | N/A | 15 |
|  | 960x720 |  |  |  | 15 |  |
|  | 960x544 |  |  |  |  |  |
| 2048x2048 | 2048x2048 |  | 25 | 25 | N/A | 25 |
|  | 2048x1536 |  |  | 27 | 25-CEILING 27-WALL | 27 |
|  | 1920x1080 |  |  |  | 27 |  |
|  | 1600x1200 |  |  | 30 | 25-CEILING 27-WALL |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  | 30-CEILING 27-WALL |
|  | 960x720 |  |  |  | 25-CEILING 27-WALL |  |
|  | 960x544 |  |  |  | 27 | 30 |
| 2048x1536 | 2048x2048 |  | 27 | 27 | N/A | 27 |
|  | 2048x1536 |  |  |  | 27 |  |
|  | 1920x1080 |  |  | 30 | 27-CEILING 30-WALL | 30-CEILING 27-WALL |
|  | 1600x1200 |  |  |  |  |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  | 30 |
|  | 960x720 |  |  |  | 27-CEILING 30-WALL |  |
|  | 960x544 |  |  |  |  |  |
| 1920x1080 | 2048x2048 |  |  | 27 | N/A | 27 |
|  | 2048x1536 |  |  | 30 | 27-CEILING 30-WALL | 30-CEILING 27-WALL |
|  | 1920x1080 |  | 30 |  | 30 | 30 |
|  | 1600x1200 |  |  |  | 27-CEILING 30-WALL |  |
|  | 1408x1408 |  |  |  | N/A |  |
|  | 960x960 |  |  |  |  |  |
|  | 960x720 |  |  |  | 30 |  |
|  | 960x544 |  |  |  |  |  |

END OF SECTION